# EK3 Feasibility Analysis

Purpose: Honest assessment of technical and commercial feasibility Date: January 2026

## Executive Summary

The EK3 concept has strong technical foundations but faces significant commercialization challenges. The core innovation (distributed coordination) is proven in software - the question is whether it translates to power electronics at competitive cost.

Verdict: Technically feasible, commercially uncertain. High-risk, high-reward.

## 1. Technical Feasibility

### What’s Proven

| Claim | Evidence | Confidence |
| --- | --- | --- |
| Distributed consensus works | Raft algorithm is battle-tested (etcd, Consul, CockroachDB) | ✅ High |
| CAN-FD handles 100+ nodes | Automotive industry deploys this daily | ✅ High |
| LLC topology at 3.3 kW | Standard design, many reference implementations | ✅ High |
| STM32G474 for power control | ST’s reference designs exist | ✅ High |
| SiC MOSFETs at this power level | Commodity parts (Wolfspeed, Infineon) | ✅ High |

### What’s Unproven

| Claim | Risk | Mitigation |
| --- | --- | --- |
| Modules sync voltage within spec | Phase drift between 100+ modules? | Droop control + tight PLL - needs hardware validation |
| Thermal management at density | 7 modules in 1U = 23 kW in small space | Forced air plenum design - needs thermal testing |
| EMC compliance at scale | 100+ switching converters = noise nightmare? | Spread-spectrum, shielding - needs EMC lab |
| Reliability at 15-25 year target | Capacitor aging, solder fatigue | Derating, redundancy - long-term unknown |

### Technical Risk Assessment

LOW RISK: - Basic power electronics (LLC, SiC, STM32) - Communication protocol (CAN-FD) - Consensus algorithm (Raft variant)

MEDIUM RISK: - Parallel operation stability (droop control tuning) - Thermal management at high density - EMC compliance

HIGH RISK: - Cost target ($50/kW) - see commercial section - 15-25 year reliability claim - unverifiable without field data

## 2. Commercial Feasibility

### The Core Problem

Established competitors have: - Existing manufacturing at scale - Certified products (CE, UL, etc.) - Customer relationships - Service networks - 10+ years of field data

EK3 has: - Novel architecture (unproven in market) - No certifications - No manufacturing - No customers - No field data

### Cost Analysis

Target: $50/kW (aggressive) ABB/Siemens: ~$100-150/kW at scale

| Component | Cost @ 1 unit | Cost @ 1000 units | Cost @ 100K units |
| --- | --- | --- | --- |
| SiC MOSFETs (4x) | $80 | $50 | $30 |
| Transformer | $40 | $25 | $15 |
| Capacitors | $30 | $20 | $12 |
| STM32G474 | $8 | $5 | $3 |
| PCB + assembly | $60 | $35 | $20 |
| Enclosure | $30 | $20 | $12 |
| Magnetics | $25 | $15 | $10 |
| Misc (connectors, etc.) | $20 | $15 | $10 |
| Total BOM | $293 | $185 | $112 |
| Per kW | $89/kW | $56/kW | $34/kW |

Reality check: $50/kW is achievable at 100K+ unit volumes. At startup volumes, more like $90-150/kW - not cheaper than incumbents.

### Certification Costs

| Certification | Cost | Time |
| --- | --- | --- |
| CE marking (EU) | €30-50K | 6-12 months |
| UL (North America) | $50-100K | 6-12 months |
| ISO 15118 (vehicle comms) | €20-30K | 3-6 months |
| Grid codes (V2G) | €50-100K | 12-18 months |
| Total | €150-280K | 18-36 months |

This is per product variant - changes require re-certification.

### Market Entry Barriers

- Chicken-and-egg: Need volume for low cost, need low cost for volume

- Risk aversion: Fleet operators won’t bet on unproven technology

- Service network: Who repairs a module in Hamburg at 3 AM?

- Liability: If charging fails and bus doesn’t run, who pays?

## 3. Competitive Analysis

### Why Would Anyone Buy EK3?

| Advantage | Real? | Competitor Response |
| --- | --- | --- |
| Lower cost | Only at massive scale | Incumbents can cut margins |
| Modularity | Yes, genuine | Kempower already modular (20 kW units) |
| No single point of failure | Yes, genuine | Add redundancy to existing designs |
| V2G ready | Yes, but so are others | ABB, Siemens have V2G products |
| Field serviceability | Yes, genuine | Training + spare parts solves this |

### Honest Competitive Position

EK3 is genuinely novel in: - Extreme modularity (3.3 kW vs 20+ kW) - Distributed intelligence (no central controller) - Fleet logistics model (swap vs repair)

But these advantages matter only if: - Cost is competitive (not at startup scale) - Reliability is proven (takes years) - Customers trust a new vendor (takes references)

## 4. Scenarios

### Optimistic Scenario

- Land pilot with progressive fleet operator (2026)

- Prove reliability over 12 months (2027)

- Accelerator funding enables scale (2027)

- EU mandate pressure creates demand (2028+)

- Reach 10K+ unit volumes, cost becomes competitive

Probability: 15-20%

### Realistic Scenario

- Struggle to get first pilot (risk aversion)

- Burn through funding on certifications

- Pivot to component/licensing model

- Sell technology to incumbent, not products

Probability: 50-60%

### Pessimistic Scenario

- Can’t achieve cost targets at low volume

- Incumbents copy modular approach

- Run out of funding before traction

- Technology becomes open-source/abandoned

Probability: 25-30%

## 5. What Would Change the Analysis

### Positive Catalysts

- Strategic partner - OEM or utility takes equity stake, provides volume guarantee

- Regulation change - EU mandates V2G capability (EK3 native advantage)

- Incumbent failure - Major charger reliability scandal creates opening

- Cost breakthrough - SiC prices drop 50%+ (happening gradually)

### Negative Catalysts

- Kempower succeeds - Modular approach proven by well-funded competitor

- Solid-state batteries - Faster charging = less need for distributed architecture

- Grid hardening - Utilities solve capacity issues, V2G less valuable

- Recession - Fleet electrification delays, funding dries up

## 6. Honest Conclusion

### What’s Real

- The technical architecture is sound

- Distributed coordination is a genuine innovation

- Long-term cost advantage is plausible at scale

### What’s Uncertain

- Path from prototype to scale manufacturing

- Customer willingness to adopt unproven technology

- Whether cost advantage materializes before funding runs out

### What’s Needed for Success

- Patient capital - 5-7 year horizon minimum

- Pilot customer - Someone willing to take risk for innovation

- Certification funding - €200K+ before first commercial sale

- Manufacturing partner - Can’t build 100K units in garage

### Recommendation

For side project: Continue - technical learning is valuable regardless of commercial outcome.

For startup with co-founder: Viable, but eyes open. Need: - €500K-1M minimum to reach first revenue - 3-5 year runway expectation - Clear pilot customer commitment before scaling - Acceptance that pivot is likely (licensing, components, acqui-hire)

The co-founder question: Is your potential co-founder ready for: - 3-5 years of grind before knowing if it works? - High probability (~60%) of pivot or exit, not original vision? - €200K+ spent on certifications before selling anything?

If yes - it’s a legitimate deep-tech bet. Many successful companies started with worse odds.

If looking for faster path to revenue - this isn’t it.

## Appendix: Key Assumptions

| Assumption | If Wrong |
| --- | --- |
| SiC prices continue falling | Cost targets impossible |
| EU mandates stay on track | Market smaller than projected |
| V2G becomes valuable | Major differentiator disappears |
| Bus fleets electrify as planned | Primary market shrinks |
| No breakthrough in battery tech | Fast charging less critical |

This analysis is intentionally conservative. Optimism is easy; honest assessment is useful.